Water & Sewer

Connection Information and Instructions

For Homeowners and Contractors

2022

Town of LeRay

Table of Contents

PACKET INTRODUCTION	1

Responsibilities	1

SEWER & WATER REQUIREMENTS	2

General Requirements	2

Technical Requirements	3

Gravity Sewer Lateral Requirements	3

Low Pressure Sewer Lateral Requirements	4

Submittal Requirements	4

Inspection Requirements	4

CONNECTION AND FIRE FLOW SERVICE CHARGE FEES	13

EASEMENT GRANT	14

Recitals	14

1.	Grant of Easement	14

2.	Description	15

3.	Ingress and Egress	15

4.	Construction and Maintenance of Facilities	15

5.	Use of Easement Premises by Grantor	15

6.	Warranty of Title and Payment	15

7.	Running of Benefits and Burdens	16

8.	Testing Authorization	16

WATER/SEWER SERVICE APPLICATION	17

Instructions	17

General Information (to be completed by the customer)	17

Owner’s Certification (to be completed by the customer)	18

TECHNICAL REVIEW	19

Sketch of Property with Water/Sewer Connection	19

## PACKET INTRODUCTION

The purpose of this procedure is to describe the application process and the basic technical requirements for the construction and installation of new water and/or sewer system components.

This package contains important information for property owners and contractors on requirements for obtaining a Water and/or Sewer Service Permit to establish a new connection within the Town of LeRay.

This document was developed to explain the Board’s requirements and provide guidelines for property owners and contractors. All required forms are included in this document.  Please read through this document carefully.  Submittal of a complete application form will help ensure that your permit is approved in a timely manner.

### Responsibilities

New water/sewer customers are responsible for:

- Completing a Water/Sewer Service Application and obtaining a Water/Sewer Service Permit which enables them to connect to one of the Town Water or Sewer District Systems. - Hiring an approved contractor to install the water/sewer system components. - Scheduling an inspection of the system components prior to connecting into the Town District. - Executing an easement, if required. - Paying all construction, engineering, easement filing, and inspection fees associated with the new water/sewer service(s).

NOTES:

- The Town of LeRay, as the System Owner, is responsible for administering and overseeing this procedure. - The Development Authority of the North Country (DANC) is responsible for operating and maintaining the Town’s Water and Sewer Systems. As the system operator, DANC personnel are responsible for ensuring that new connections are constructed and installed in accordance with applicable design standards. DANC personnel are also responsible for documenting new connections and distributing details to the system owner.

Record of Changes

| Revision Date | Description of Changes |
| --- | --- |
| 5/01/08 | Original Document Created |
| 06/01/22 | Revised Document |

## SEWER & WATER REQUIREMENTS

### General Requirements

| ☐ | - The Town can only provide service to customers that are located within an existing water or sewer district. |
| --- | --- |
| ☐ | - Due to financing stipulations, municipalities within the Route 3 Sewer Corridor require that new sewer services are turned over to the municipality to operate and maintain. |
| ☐ | - If you are constructing a new sewer service lateral in the Town of LeRay Sewer District # 3, it is required that your service be turned over to the municipality after construction, then an updated easement showing the location of any new system components must be filed with the County. This easement provides the Town with proper access to maintain the sewer system components. The customer is responsible for all fees associated with the execution and filing of new easements. The Town will file the easement with the County. |
| ☐ | - Requests for connection into the Town of LeRay water and/or sewer districts must be submitted by completing a Town of LeRay Water/Sewer Service Application. |
| ☐ | - Connections must be designed and constructed in accordance with the original criteria identified in the contract documents and specification relating to the respective system, unless the proposed criteria can be shown to be more advanced and of better quality than that of the original specification. Contract documents are available at the Town offices, and at the Development Authority of the North Country’s Warneck Pump Station located at 23557 NYS Route 37, Watertown, NY 13601. |
| ☐ | - Customers must utilize a contractor approved by the Town and/or Route 3 Sewer Board for the installation of any water/sewer system components. The following contractors have been approved by the Town and the Route 3 Sewer Board: |

Contractors Approved to Perform Direct Connections to Sewer Mainlines and Lateral Installations:

| Contractor | Address | Phone # |
| --- | --- | --- |
| Bach & Co. | 11176 County RTE 9, Clayton, NY 13624 | (315) 686-3083 |
| Cunningham Excavation | 3592 Cobblestone Dr., Cazenovia, NY 13035 | (315) 655-5800 |
| Thomas Excavating, LLC | 17523 Doxtarer St., Adams, NY 13605 | (315) 232-2162 |
| North Country Contractors LLC | 12141 Whitney Rd., Henderson, NY 13650 | (315) 938-7080 |
| Marcellus Construction | 4615 Crossroads Park Dr., Liverpool, NY 13088 | (315) 451-6655 |
| Syracuse Utilities | 17668 Old Rices Rd., Watertown, NY 13601 | (315) 256-6591 |
| W.D. Malone | 708 County RTE 7, Hannibal, NY | (315) 561-6784 |
| Steven J. Hall | 3128 NY-12E, Cape Vincent, NY 13618 | (315) 654-3691 |

| ☐ | - All new connections must include a means for metering in order to accurately bill for the services provided. Residential sewer customers must be a current water customer or must install a meter on their private well. Meters will be provided and installed by the Town Operator. If the new structure does not have a basement, then a separate meter pit will be required. Details for Typical Basement Meter Installation and Typical Meter Pit Installations are included in the Instruction Packet. (page XXX) |
| --- | --- |
| ☐ | - Most water/sewer service determinations will be made within 30 days of receipt of a complete Town of LeRay Water/Sewer Service Application. |

### Technical Requirements

| ☐ | - Technical requirements for water service are presented in the Town of LeRay Water Service Law. Some basic installation details have been included for reference with the Instruction Packet. Customers may reference these details to determine acceptable pipe material, covering, backfill, etc. |
| --- | --- |
| ☐ | - Detailed technical requirements for sanitary sewer systems are presented in the Town of LeRay Sewer Use Law and the latest Contract Documents and Drawings for the Route 3 Sewer Corridor. Customers may reference these documents to determine acceptable pipe material, covering, backfill, etc. |
| ☐ | - More complex systems may require that the customer hire an engineer licensed by the State of New York to complete the design. Detailed information and drawings are available by requesting a copy of the Route 3 Contract Documents and Drawings from DANC at (315) 782-8661. |

### Gravity Sewer Lateral Requirements

| Some basic requirements for single family gravity sewer laterals are listed below and are included with more detail in Section 02733 of the Contract Documents and the Sewer Installation Details found in this document: |  |
| --- | --- |
| ☐ | - The minimum horizontal distance between water and sewer line shall be 10’ except when lines must cross. At intersections water and sewer lines shall be separated by a minimum of 18 vertical inches. |
| ☐ | - Water and sewer lines shall not share excavation trenches with other utilities and must be separated by a minimum distance of 6’ whenever possible. |
| ☐ | - Laterals shall be installed with a minimum of 4’ of cover. Where it is physically impossible to provide cover of 4 feet, the depth may be reduced to a minimum of 3 feet and the pipe shall be insulated, as approved by the Town. |
| ☐ | - Magnetic marking tape 2 inches wide with the words SANITARY SEWER BELOW shall be installed not more than 2 feet below finished grade for all sewer laterals. |
| ☐ | - Gravity sewer laterals shall be 4” PVC Plastic SDR 35 pipe and fittings. Pipe shall be joined with integral bell and spigot rubber gasketed joints. |
| ☐ | - 4” service laterals shall be installed at a minimum of ¼” per 1’ slope. |
| ☐ | - Service laterals shall be connected to existing sewer main lines through the use of a machine drilled hole using equipment expressly made for this purpose. Laterals shall be connected to the sewer main through the use of an Inserta Tee Fitting made for PVC SDR 35 pipe. Fittings are available by contacting the Development Authority or the manufacturer (Inserta Fitting Co., Tel. 503/357-2110). |
| ☐ | - Sewer laterals shall not be connected directly to manholes (Sewer District # 3 only). |
| ☐ | - Cleanouts shall be provided at least every 100’; at each change in direction of the line; and within 10 feet of the house foundation (and at road ROW in Sewer District 1 & 2). |
| ☐ | - For normal soil conditions, the trench shall be excavated 4 inches deeper than the bottom of the pipe. Pipe bedding material used shall be NYSDOT Subbase Course 304 Type 2 crushed stone. |
| ☐ | - Backfill material shall be free of roots and vegetation and shall have no stones greater than 4 inches in diameter. |
| ☐ | - Pipe bedding shall be placed in maximum lifts of 8 inches after the pipe is joined. |
| ☐ | - Do not compact directly over PVC pipe until backfill has reached 2 feet above top of pipe. |

### Low Pressure Sewer Lateral Requirements

| Some basic requirements for single family gravity sewer laterals are listed below and are included with more detail in Section 02733 of the Contract Documents and in the Sewer Installation Details found in this document: |  |
| --- | --- |
| ☐ | - Gravity sewer laterals that cannot meet minimum slope requirements will require a grinder pump. |
| ☐ | - Grinder pumps will be supplied by DANC on behalf of the Town and must be installed by an approved contractor. |
| ☐ | - The grinder pump must be paid for by the property owner before it can be supplied to the contractor. The property owner will pay the Town for the cost of the pump. These funds will be forwarded to DANC by the municipality with the completed Sewer Service Application. Once DANC receives payment the pump can be released to the contractor for installation. |
| ☐ | - The property owner’s contractor must install the grinder pump in accordance with Section 11308 of the Route 3 Contract Documents for the Route 3 Sewer System, if applicable, and E-One Grinder Pump Installation Instructions. Copies of these documents are available upon request from DANC. |

### Submittal Requirements

| ☐ | - All new service connections must be reviewed and approved by the Town, their Engineer and/or the Development Authority. |
| --- | --- |
| ☐ | - The property owner must submit a completed Town of LeRay Water/Sewer Service Application to the Town Clerk at the Municipal Office. |
| ☐ | - The application must be submitted with the proper connection fee and a Location Sketch. |
| ☐ | - The location sketch must show the property boundaries, buildings, approximate length of service lateral with reference dimensions to two permanent structures, location of cleanouts, tie-in point to mainline, and any other relevant information. An example of an acceptable Location Sketch is attached for reference. |
| ☐ | - All new service connections must be reviewed and approved by the Town, their Engineer and/or the Development Authority. |
| ☐ | - Certain new sewer service connections require that services be turned over to the Town for operation and maintenance after construction in accordance with the Town’s Sewer Use Law. As such, the Customer will be required to execute a property easement providing access to the municipality. Customers should contact the Town Clerk to complete the easement process. A standard Town of LeRay Water/Sewer Easement is attached for reference. This easement must be executed and submitted with the applicable County filing fee prior to obtaining approval for connection. |

### Inspection Requirements

| ☐ | - All approved connection requests must be inspected by a DANC representative. The customer or their representative must schedule the connection with the Development Authority at least 48-hours prior to the connection. Customers must have an approved Town of LeRay Water/Sewer Service Application on file with the Development Authority prior to scheduling an inspection. If the connection is scheduled outside normal business hours or the inspection takes longer than two hours, additional charges may be applied. To schedule an inspection, contact the Development Authority of the North Country at 315-782-8661. |
| --- | --- |

## CONNECTION AND FIRE FLOW SERVICE CHARGE FEES

| Connection Fees |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All Water and Sewer Districts |  |  |  |  |  |  |  |
| District |  | Water |  |  | Sewer |  |  |
| Single Family Residence (R-1) |  | $45.00 (plus meter) |  |  | $150.00 (plus meter) |  |  |
| All Other Districts (including commercial, industrial, institutional, multi-family, and any other uses) |  | - $500.00 plus meter (and out of pocket costs, if any) for up to 1” service) - $1,000 plus meter (and out of pocket cost, if any) for larger than 1” service |  |  | - $500.00 (and out of pocket costs, if any) for up to 6” lateral service) - $1,000 (and out of pocket cost, if any) for larger than 6” lateral service |  |  |
| Water Meter Purchase (subject to price changes from supplier) |  |  |  |  |  |  |  |
| - $384.00 Sensus ¾” radio-read with hardware - $420.00 Sensus 1” radio-read with hardware - Sensus 1 ½” or larger will have to be ordered - All others will be special-ordered – actual cost |  |  |  |  |  |  |  |
| Reconnect Fees – Water |  |  |  |  |  |  |  |
| - $35.00 during normal operator working hours - $75.00 outside normal operator working hours |  |  |  |  |  |  |  |
| Fire Flow Service Charges (for sprinkler systems in all non-welding structures) |  |  |  |  |  |  |  |
| Size of Water Line | 2” | 3” | 4” | 6” |  | 8” | 10” |
| Quarterly Charge | $20.00 | $40.00 | $60.00 | $100.00 |  | $150.00 | $250.00 |

## EASEMENT GRANT

This Easement Grant is made this _______ day of _____________, 20_____ between ___________________________________ residing at__________________________________________ ("Grantor", whether one or more) and the TOWN OF LERAY, a municipal corporation of the State of New York, having an office at 8650 LeRay Street, Evans Mills, New York 13637 (the "Grantee").

### Recitals

- Grantee is in the process of building, installing, or maintaining a:

  - wastewater collection system, including mains, laterals, pump stations, meters, and other appurtenant devices; and

  - water supply lines, including laterals and appurtenant facilities, including pumps and meter for water (hereinafter referred to as "Facilities").

- Grantor is the owner of certain real property within the service area of the facilities known as the ________________________ tax map number _______________________.

- A portion of the facilities need to be installed on Grantors property in accordance with certain engineering plans drawn by ________________________________, as shown on Schedule "A" annexed.

- Grantee is seeking an easement to install such facility.

- Grantor is willing to provide and easement for such facilities.

In consideration of One Dollar ($1.00) and other good and valuable consideration, the receipt and sufficiency of which are hereby acknowledged, the following grants, agreements, covenants and restrictions are made:

#### Grant of Easement

  - Permanent Easement

Grantor hereby grants and conveys to Grantee, its successors and assigns, a twenty (20) feet wide permanent right-of-way and easement on, over, under, across and through the premises of Grantor as described below ("the Permanent Easement") for the lawful purposes of the Grantee including laying, relaying, constructing, reconstructing, maintaining, inspecting, operating, cleaning, repairing, replacing and removing sewer mains, laterals or other appurtenant facilities, including supporting apparatus, meters and structures, all other facilities appurtenant thereto and any other lawful improvements thereto (collectively "the Facilities") and making such excavations and performing all work as, in the sole discretion of Grantee, its successors or assigns, may be necessary or desirable.

  - Temporary Easement

During and for the purpose of the construction and restoration of any Facilities and appurtenances thereto, the Grantee shall have the right to clear and use additional land ten (10) feet in width on each side of the Permanent Easement (the "Temporary Easement").  The grant of Temporary Easement shall terminate upon completion of construction of the facilities and surface restoration.  Excepting and reserving from the permanent and temporary easements any area of land upon which there is situated a part of a permanent structure or building on the date of this easement grant.

#### Description

The parties hereto agree that "as built" drawings shall be filed with the County Clerk to show the precise location of the facilities and that the "as built" drawings aforementioned shall constitute the actual location of said easements as hereinbefore set forth.

#### Ingress and Egress

The Grantee, its agents, contractors, and their workmen shall have the right to reasonably cross and re-cross the remaining adjacent lands of the undersigned on foot or with motor vehicles and their equipment for all purposes set forth in this Easement Grant and incidental thereto, including but not limited to the construction, inspection, cleaning, operation, maintenance, and repair of the Facilities.

#### Construction and Maintenance of Facilities

- Upon completion of the construction of the Facilities, as well as after any subsequent repair, reconstruction, replacement, or any other work in connection with the Facilities, Grantee, at its own cost and expense, shall restore the surface of the easement to, (as nearly as is reasonably practicable), their condition prior to the commencement of the work.

- Grantee shall defend, hold harmless and indemnify Grantor of and from all damages or judgments arising from injury or death to person or property, except such damages to the property of Grantor arising from the use of the Permanent Easement by Grantor pursuant to paragraph 4, by reason of and caused by the actions of Grantee in carrying out the purposes of this Easement Grant.

- The Grantee may during construction and thereafter from time to time, trim, cut and remove by manual, mechanical or chemical means, brush, trees, vegetation, and other physical obstructions on the easements.

#### Use of Easement Premises by Grantor

The right to use the Permanent Easement for any purpose not interfering or inconsistent, in the opinion of Grantee, with this Easement Grant is expressly reserved by Grantor, provided, however, that Grantor shall neither build, erect, construct or place upon, over or under the Permanent Easement, anything of a permanent nature or character, except that Grantor may maintain and replace anything currently existing.  Furthermore, the Grantor will not impound water nor do or cause to be done, directly or indirectly, any damage to, interference with, or create any potential hazard to the Facilities, and will not remove soil from the Permanent Easement so as to leave the Facilities nearer to the surface of the land than when laid, nor in any way interfere with or disturb the grade of the soil as completed by the Grantee.  It is further understood and agreed that the Facilities shall at all times be and remain the property of Grantee and under its unconditional control and supervision.

#### Warranty of Title and Payment

Grantor has by this Easement Grant provided the Grantee with good and marketable title to the easements.  The Grantor hereby warrants ownership in fee simple of the easements, has a good right to convey and grant the foregoing easements, that the easements are free and clear of all liens and encumbrances and will forever warrant title.  Furthermore, that in compliance with Section 13 of the Lien Law, the Grantor will receive the consideration for this conveyance and will hold the right to receive such consideration as a trust to be applied first for the purpose of paying the cost of the improvement and will apply the same first to the payment of the cost of the improvement before using any part of the total of the same for any other purpose.

#### Running of Benefits and Burdens

All provisions of this instrument, including the benefits and burdens, run with the land and are binding upon and inure to the heirs, assigns, successors, lessees, tenants and personal representatives of the parties thereto.  However, the Grantee, its successors and assigns shall have the right to assign to others, in whole or in part, any or all of the Grantee's rights, privileges and interests in this Grant.

#### Testing Authorization

Grantor hereby grants Grantee the right, upon reasonable notice, before, during and after construction, to test the quality and quantity of Grantor's water supply and to photograph and inspect the exterior and interior of Grantor's structures which may in any way be affected by construction of the Facilities.

IN WITNESS WHEREOF, the Grantor has executed this Easement Grant as of the day and year first above written.

#### CERTIFICATE OF ACKNOWLEDGEMENT BY NOTARY

| X |  | X |
| --- | --- | --- |
| Signature of person appearing before the Notary |  | Signature of person appearing before the Notary |
| X |  | X |
| Signature of person appearing before the Notary |  | Signature of person appearing before the Notary |

STATE OF NEW YORK      		)

) ss.:

COUNTY OF JEFFERSON    	)

On this ____________ day of ________________ in the year __________ before me came ______________________________________________, to me know to be the individual(s) in and who executed the foregoing instrument and acknowledged that he/she/they executed the same.

X

Notary Public Signature

## WATER/SEWER SERVICE APPLICATION

| OFFICE USE ONLY |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Is a new/updated property easement required? |  |  |  |  |  |  |  | ☐ Yes ☐ No |  |  |  |  |  |
| (If an easement is required, it must be executed prior to connection approval) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water Service Size: |  |  |  |  |  | Connection Fee: ☐ $450.00 ☐ $500.00 ☐ $1,000.00 |  |  |  |  |  | Meter Cost: | $ |
| Sewer Lateral Size: |  |  |  |  |  | Connection Fee: ☐ $150.00 ☐ $500.00 ☐ $1,000.00 |  |  |  |  |  | Total Due: | $ |
| ☐ Cash |  | ☐ Check |  |  | Check #: |  |  |  |  | Receipt #: |  |  |  |
| Date submitted: |  |  |  |  |  |  |  |  |  | Received by: |  |  |  |
| Municipal approval that applicant is eligible for new service connection to the Town System |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  | Signed: |  |  |  |  |  |

### Instructions

All information must be typed or legibly printed.

All sections of the form must be completed to avoid delays in processing.

The property owner must sign the form.

Submit completed form with payment and Location Sketch to the Town Municipal Office.

The Clerk will forward the application for Technical Review.

You will be notified by phone if your permit has been approved or if your permit is missing Information required to properly review. If your application is denied you will be notified in writing as to the reason.

### General Information (to be completed by the customer)

| Property Owner: |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Email: |  |  |  |  |  |  |  | Phone: |  |  |  |  |
| Owners Address: |  |  |  |  |  |  |  |  |  |  |  |  |
| City: |  |  |  |  | State: |  |  |  | Zip: |  |  |  |
| Tax Parcel #: |  |  |  |  |  |  | ☐ Residential |  |  |  | ☐ Commercial |  |

| Job Address: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mailing Address: |  |  |  |  |  |  |  |
| City: |  |  |  | State: |  | Zip: |  |

| Approved Contractor: |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Email: |  |  |  |  |  |  | Phone: |  |  |  |  |
| Mailing Address: |  |  |  |  |  |  |  |  |  |  |  |
| City: |  |  |  |  | State: |  |  | Zip: |  |  |  |
| Is the requested connection to serve an existing structure? |  |  |  |  |  |  |  |  |  |  | ☐ Yes ☐ No |
| If yes, is the requested connection to relieve a failing well or septic? |  |  |  |  |  |  |  |  |  |  | ☐ Yes ☐ No |
| If the connection is to serve a new structure, what is the anticipated occupancy date? |  |  |  |  |  |  |  |  |  |  |  |
| *A sketch of the new service connection is attached. |  |  |  |  |  |  |  |  |  |  | ☐ Yes |

### Owner’s Certification (to be completed by the customer)

Application is hereby made to obtain a permit to do the work and installation as indicated. I certify that I am the Owner of the property located at _____________________________________________ and that I am requesting a permit to install a new water/sewer service.

In consideration of the granting of this permit, I agree to the following:

That no work or installation has commenced prior to the issuance of a permit;

That all work will be performed to meet the standard of all applicable State and Local laws regulating construction;

To maintain the water service lateral in accordance with the Town’s requirements;

To schedule an inspection of the service with DANC before any portion of the work is covered and prior to connecting into the main water or sewer line.

| Date: |  |  | Signed: |  |
| --- | --- | --- | --- | --- |

## TECHNICAL REVIEW

(To Be Completed By DANC)

| Application Reviewed By: |  |  |  |  |  |  |  | Date: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Application Action: |  | ☐ Approved, as submitted |  |  |  |  |  |  | ☐ Approved with conditions |  |  |
|  |  | ☐ Returned, due to incomplete submittal |  |  |  |  |  |  | ☐ Denied (see attached) |  |  |
| Name: |  |  |  |  | Property Address: |  |  |  |  |  |  |

### Sketch of Property with Water/Sewer Connection

Be sure to show reference points and distances as accurately as possible.